Lab 4

Objective:

Create a one-bit adder

Create a three-bit adder using one-bit adders as symbols (modular design)

Design:

Table 1 - One-bit Adder Truth Table

Table 2 - Three-bit Adder Test Values

Figure 1 – Cout K-Map and Function			         Figure 2 – SUM K-Map and Function

Figure 3 – One-bit adder Circuit

Figure 4 – Three-bit adder Circuit

Procedure:

Using Xilinix ISE:

One-bit Adder:

- Construct one-bit adder schematic from prelab diagram (Fig. 3)

- Compile schematic

  - Troubleshoot if necessary

- Run behavioral simulation for all 7 inputs

  - Print results (Fig. 5)

- Run Post-Route Simulation (adds time delays)

  - Print results (Fig. 6)

- Download the design to the board, following previous lab’s steps (Lab 3, steps 17 through 19-XI)

  - Have outputs Cout and Sum each go to an LED

  - Have inputs A, B, and Cin accept input from physical switches

- Test the downloaded circuit against the prelab truth table (Table 1)

Three-bit adder:

- Create a schematic symbol from the one-bit adder schematic

  - Highlight the one-bit adder schematic file from the design window

  - Run “Create Schematic Symbol” in the process window

- Create a three-bit adder schematic using one-bit adder symbols

  - Create a new schematic file named “three-bit adder”

  - Select the “Symbol” tab

  - In the “Categories” section, select the project directory

  - Select the “one-bit adder” symbol

  - Place 3 “one-bit adder” symbols on the schematic

  - Connect the 3 Symbols appropriately

    - Connect the first symbol’s “Cout” to the second symbols “Cin”

    - Connect the second symbol’s “Cout” to the third symbols “Cin”

  - Add I/O markers

    - A0, B0 and Cin to the input of the first symbol

    - SUM0 to the output of the first symbol

    - A1 and B1 to the input of the second symbol

    - SUM1 to the output of the second symbol

    - A2 and B2 to the input of the third symbol

    - SUM2 and Cout to the output of the third symbol

  - Run behavioral simulation for given test inputs (Table 2)

    - Run each input for 5ns

    - Print results (Fig. 7)

  - Run Post-Route Simulation for first 4 test inputs (Table 2)

    - Run each input for 20ns

    - Print results (Fig. 8)

  - Download the schematic to the FPGA, similarly to the one-bit adder

    - Connect the 7 inputs to switches

    - Connect the 4 outputs to LEDs

  - Test the board with the test inputs (Table 2)

Data:

Figure 5 – One-bit Adder Behavioral Simulation

Figure 6 – One-bit Adder Post-Route Simulation

Figure 7 – Three-bit Adder Behavioral Simulation

Figure 8 – Three-bit Adder Post-Route Simulation

Data Analysis:

Each of the simulations (Figs. 5-8) and downloaded circuits matched the calculated truth tables (Tables 1 and 2). There was an issue where the three-bit adder was not matching the truth values expected. Troubleshooting showed that the Cout and SUM2 outputs were switched and correcting that gave the expected results.

Conclusion:

The goal of this experiment was to construct bit adders as well as observe the benefits of modular designs in increasingly complex circuits. Overall, the one-bit adder circuit seemed to become relatively complex for what seems to be a simple task, but it gets the job done. Constructing a three-bit adder from scratch in a similar fashion to the one-bit would begin to get prohibitively complex. By being able to break down the more complex end product (three-bit adder) into more manageable modules (one-bit adders) reduces the complexity significantly and makes the final design much faster to assemble.